# Fabrication and Integration Process

### 1. Material Choice Justification :

- Cardboard: The primary structure of the physical setup is made using cardboard due to its accessibility  and ease of fabrication. It allowed for quick prototyping, easy cutting, and making modifications.

-Foam board is used as a supporting base beneath the cardboard to provide additional structural stability and rigidity and hide all the wired and breadboard. The foam board reinforces the structure without significantly increasing overall weight, and gives stability to the cardboard incase it bends.

The Sphinx head and body are fabricated using 3D printed PLA. PLA is chosen for its precision, allowing detailed and complex forms to be produced that would be difficult to achieve with manual materials. It also provides a clean and finished aesthetic, enhancing the visual quality of the artefact while remaining lightweight and easy to print.

2. Fabrication Steps :

Fabrication Steps (Build Log)

- Concept planning and layout sketching of the overall Sphinx game structure and logic

- Cutting cardboard panels for base, walls, and enclosure using manual tools

- Reinforcing base structure by mounting foam board beneath cardboard

- Assembling main enclosure using adhesive (glue/tape)

- Designing Sphinx head and body digitally for 3D printing

- 3D printing Sphinx components using PLA

- Post-processing printed parts (cleaning, minor sanding)

- Mounting servo motor inside structure for head and door movement

- Fixing Sphinx head onto servo shaft for motion

- Installing door mechanism and aligning with servo motor

- Placing and securing LEDs into front panel for progress display

- Attaching NeoPixel strip inside structure for ambient lighting

- Wiring all components (LEDs, touch pins, buzzer, servos) to ESP32

- Setting up capacitive touch wires for user input (A/B/C/D)

- Connecting ultrasonic sensor at entrance for player detection

- Testing individual components

- Integrating full system and debugging interactions

- Hosting web interface and testing real-time game feedback

- Final assembly, cable management, and enclosure closing

- Final testing and calibration of touch sensitivity and servo motion